Mike Schmidt, Project Engineer

February 25, 2013

- Tony Puttnam would problem-solve and could return to the archives and find consistency with Wright’s plans

- By referendum, the project had to follow 1959 revision

- Footprint = same

Foundation and Sitework:

- Lakeside wall of Ballroom follows lakeshore

- Front edge of lake wall extends 18” below lake surface

- Bike lane was maintained through construction

- DNR required silt curtain to keep runoff from entering Lake Monona

- Building is about 1 block long

- Parking 2 blocks long

- Ice would close after cut open, which presented challenges

- Code didn’t dictate how to design for ice force, had a study done

- Piles designed for 120 tons, 330 proven

- Exhibition Hall supported on piles as well, 30 ft high to as tall as 90 ft

- Started driving piles in ice, marked where to put them

- Pile Dynamic Analyzer – measures stress – determine capacity of pile

- Piles sometimes free fall 20 ft. when there is no substrate underneath.  If you drive 60 blows for 6 inches, you feel better because the piling is becoming secure in the bedrock. They are able to gauge because the metal they are impacting in the driving process starts to deform.

- Construction started Dec ’94 – funding coming through that year

- Barges and boats used in spring when ice thawed

- Piles were often built in clusters

- Reinforcement bars to resist ice force

- Lakeside Commons about 5 ft over lake: still above flood stage

- Deck – plank platform to work form

- Pour slab over planking

- Rip-rap – manhole covers on bikepath give access to riprap. Sometimes shifts needs to be fixed.

- Water quality –South side of Capitol Square dumps stormwater, this had to be redirected around building

- Vortex – filters contaminants from stormwater

Building Superstructure

- 101 ft clear span from Meeting Rooms Wall to terrace

- Exhibition Hall wall 35’ high

- Property Line on Wilson St threw building off 4 degrees/not perpendicular to MLK Jr.

- Scalloped pre-cast pieces were hung from cantilevered steel beams

- Fiberglass reinforced concrete – 1.5 in thick

- 2 vibration consultants hired

- Fireproof trusses

Parking Structure

- Panels from a company in Burlington, WI

- “Double-ts” refers to the pre-formed shapes of concrete found in the ramp.

Helix

- Floors sloped to inside for drainage

- East Helix contains:

- 1st floor-chillers

- 2nd floor- electrical vault

- Top=cooling tower

- West Helix is empty

- Floors cantilevered from columns in center

- Ballroom sculpted forms- preformed offsite

Rooftop

- Fountain is supported by elevator

- Roof system 1st time used in U.S., though it had been previously used in Europe

- Black tan membrane applied, then insulation, then adjustable pedestals

- Aircraft cables keep planters in place

- Drainage in each planter

- One place to see pilings above ground is in parking ramp by Hilton at lake level.

- Pilings do not correspond with or relate to columns in Grand Terrace. However, columns with steel beams are continuous from Level 1-4.  (i.e. can be seen in Lakeside Commons, Community Terrace and Grand Terrace)

- Netting curtain was removed after 1st year – fish were getting caught

- “Rip-rap” refers to an uneven surface of rock and concrete pieces that protect the shoreline.  Rip-rap is under the building.

- Ice force – thermal expansion on sunny day

- Spring break-ups – wind-mooring ice

Jeff Griffith

Monona Terrace Facility Supervisor

- USGBC – formed in 1993

- Buildings use 39% of all energy consumed in the US, 74% of all electricity

- 5 billion gallons of water used to flush toilets in U.S.

- Our certification is LEED for Operations & Maintenance (previously called LEED Existing Building)

- LEED certification is measured on these criteria:

- Site Development – example: Sand is used instead of salt for snow and ice

- Water Savings – example:  We use drip irrigation wherever we can

- Water Efficiency – example:   Motion sensor fixtures in bathroom for soap, towels, etc. In last year, we decreased water usage by 50%. New bathroom fixtures will be even more efficient.

- Energy & Atmosphere – Eliminate chlorofluorocarbons in chiller system. Hot water produced by natural gas.  Scheduled climate control system for rooms.   Total energy use decreased by 34% over last 5 years!

- Indoor Air Quality

- Green cleaning products  contain low volatile organic compounds (voc’s).  Fore the exterior of the building, we powerwash with water and use no chemicals.

- How much outdoor air do you bring in?  The more, the better.  The trade-off is using more energy to cool or heat that air.

- Integrated pest control used

- Pesticides applied once in 8 yrs

- Low VOC paints

- We are replacing current lighting with LED lighting (and reducing mercury) as much as possible.  1/3 changed so far. Goal = 90%

- Materials & Resources – Including purchasing & what we do with items when we’re done with them.

- Mercury in lamp stock

- Metals recycling in 2012 – 3.5 tons

- 6.5 tons – clean fill reused  (broken exterior tile like that on roof can be used for fill in road construction and other projects)

- 18 tons of palettes, 1 ton of batteries recycled in 2012

- 3.5 tons of food donated to food pantries